Task 1: To Create Main Loop, Display Prompt, Read User Input, Handle Exit Conditions, Design Control Flow Diagram, Test Interactive Loop

Task 2: Using the following system calls, copy the content from File1.txt to File2.txt.

- Open()

- Read()

- Write()

- Close()

open()

syntax: open(“ filename.txt”, modes, permissons)

Example: open(“First_Prgm.txt”, O_CREAT | O_WRONLY, 0644);

Table 1: Modes of Open system call

| Flag | Purpose |
| --- | --- |
| O_RDONLY | Open for read only |
| O_WRONLY | Open for write only |
| O_RDWR | Open for both read and write |
| O_CREAT | Create file if it does not exist |
| O_TRUNC | Truncate (empty) an existing file |
| O_APPEND | Append data to the end of the file |
| O_EXCL | Fail if file already exists (used with O_CREAT) |

,